How to convert ASi Proworx application in Unity Pro?

- For Unity Pro V8.1 install the Hotfix UnityPro_V81_HF0153314E- Converter

For Unity Pro V10 or higher this step is not needed.

- Open the .pwx (or .dcf) in Unity Pro

During conversion from the Unity Pro application: don't select "Full Topological

- Save the log file, it contains the mapping of the 140EIA92100 modules in proworx.

- After conversion open all 140EIA921000 configuration screens inside Unity Pro application, validate them, close all configuration screens, reopened them to check if the mapping changed.

EIA screen before validation: mapping on bit %I and %M

EIA screen after validation: mapping on bit %IW and %MW

- Save the application, it is ready to be adapted