How to adapt the application in Unity Pro

Example of adaptation to do on Asi Proworx application mapped on %I and %M bits converted in Unity Pro.

Unity Pro works on Words for Asi modules, the application must be adapted in order to transfer the input word received by the Asi module in input bits and to transfer the output bits after the treatement of the application in output word for EIA module. So with adding only 2 sections the application should be the same in Proworx and Unity Pro :

Application in Proworx

Application in Unity Pro :  2 sections have to be added.

This application use several EIA 921 00 modules, for easier explanation we will focus on one of the module, the treatements are the same for all EIA modules : module at the 3rd location.

- Remapping of the EIA modules

When in Proworks the input/output of these modules are mapped on %I and %M bits

After conversion in Unity Pro these modules are mapped on %IW and %MW words

IMPORTANT

The user has to find free memory in the %IW and %MW memory, the address es put by default (same address than the initial %I and%M bit memory are not obligatory free) are surely not available because used by other modules or variables.

In this example we choose the following :

So to summarize the EIA module was mapped on %I4321 and%M4321, it has been mapped now on %IW1200 and %M1200

- Symbolization and localisation of the variables

To keep easy the programmation (working on symbol and not on direct address)  we define the following variables :

Types :

Input Variables :

The input words provided by the EIA module in Unity Pro

The input bits provided by the EIA module in Proworxs (and used in the application)

Outputs variables

The output bits provided by the EIA module in Proworxs (and used in the application)

The output  words provided by the EIA module in Unity Pro

- Section to create and to add at the beginning to the application :

In addition to the copy bits into words this section make a swap (*) in order to put the bit in the right order like in Proworx.

- Section to create and to add at end of the application :

In addition to the copy words into bits this section make a swap(*)  in order to put the bit in the right order like in Proworx.

(*) the Swap is done by attached DFB, he has to be imported in the application, define only one instance of this DFB and call it Swaps, it could be used as an EF.