## Specific Function Blocks for XVU Pulse Signal

## How to Install the Libraries

The libraries are only available for the controller M241 and M258. And it is compatible with SoMachine v4.1 and SoMachine v3.1.

### SoMachine v4.1

The XVUP libraries for SoMachine v4.1 support two controllers M241 and M258 (simulated). We recommend you to use these libraries.

XVUP Modules for M241_SoM41.compiled-library

XVUP Modules for M258_SoM41_Simulated.compiled-library

### SoMachine v3.1

There is a corresponding XVUP library for M258 (simulated) for SoMachine v3.1.

XVUP Modules for M258_SoM31_Simulated.compiled-library

### How to Install (SoMachine v4.1)

- Open SoMachine v4.1 and create a new empty project. (Or open your existing project.)

- Enter the Logical Builder.

- Add a new M241 controller if this is a new created project.

- Open the menu and enter the Library Repository.

(This step only needs to be done one time.)

- Browse and add the library to the Repository.

(This step only needs to be done one time.)

Click the install button and browse the “XVUP Modules for M241_SoM41.compiled-library” file.

If it succeeds, you will find it in the “Miscellaneous” tree.

- Import the library into the project.

Switch to the Application trees tab, right click on the project name and add a Library Manager to the project.

- Click on “Add library” button to add the library to the project.

- Once done, you will find the library and four imported function blocks.

## Function Block

### M241_PWM_MULTICOLOR

This function block is used for XVU Multi-Color module. It uses the build-in PWM module in M241 controller. You have to firstly enable and configure PWM of the controller.

The definition of Command of this function block:

### M241_PWM_VOICE

This function block is used for XVU Voice module. It uses the build-in PWM module in M241 controller. You have to firstly enable and configure PWM of the controller.

The definition of Command of this function block:

### SIMULATED_PWM_MULTICOLOR

This function block is used for XVU Multi-Color module. It uses the timers to simulate the PWM pulses.

The definition of Command of this function block:

### SIMULATED_PWM_VOICE

This function block is used for XVU Voice module. It uses the timers to simulate the PWM pulses.

The definition of Command of this function block:

## How to Use

You could use the imported function blocks by the box component in Logical Builder which is very similar to the usage of other function blocks.

Select the function block you will use.

### Sample to use M241_PWM_MULTICOLOR

- Enable the PWM module in M241 controller.

- Set the value for all input pins.

Select your PWM command. It will generate different duty-ratios of PWM pulses to display different colors.

XVUPModulesM241

Select the output type: Sink or Source.

XVUPModulesM241

Select the Output pin; it must be the same as what was set in the first step.

XVUPModulesM241

- Once done, you could run the application to test.

### Sample to use SIMULATED_PWM_VOICE

- Set the value for all input pins.

Select your PWM command. It will generate different duty-ratios of PWM pulses to play different voices.

XVUPModulesM241

- Select the interface to output.

Please make sure the output pin is free to use.

You could check it below.

- Once done, you could run the application to test.

## Constrains

For usage of SoMachine, please refer to the user manual of SoMachine.

If you simultaneously import equal to or more than two XVUP libraries into one project, you have to enable the "namespace" to avoid name conflict. Please refer to the user manual of SoMachine.