How to activate the local remote key on the remote keypad

At first show that the drive is ordering via the terminal

On the first menu

Term: inform that the drive is ordering by the terminal

RDY is the status of the drive

0.0Hz: AI1 reference speed

Local remote key on the remote keypad is disactivated.

On the first  menu

Action: Set a speed reference with AI1=20%

On the first menu

Action: SET Di1=1

RUN means that the motor is running at 2.08 A by the terminal

The current is 2.09 A in the motor

On the first menu

Action:SET DI1 = 0

RDY is now the status of the drive

10Hz is the reference given by AI1

now how to order via the remote keypad

Action:Press ENTER key

Main menu is now present

Action: Select and press Enter on the Complete settings

The new menu is: complete setting

Action: Select the sub-menu 5.4 command and refere

Action: Press ENTER on Command and Referen  sub-menu

The new sub-menu  is Command and refere

AI1 means that the reference is given by AI1

HMI cmd is always is in disabled state

Action: Select HMI cmd parameter with the dial

Action: Press ENTER on HMI cmd parameter

New menu HMI cmd.

HMI cmd is disabled

Action: Select and press Bumpless   sub-menu

Now HMI cmd is activated on bumpless.

The local remote key is now activated.

On the first menu

Term is always present

On the first menu

Action:  Press local remote key present on the remote keypad to switch from TER to HMI.

AI1 reference is copied to HMI reference

HMI is now the new command of the drive

Action: Press RUN key present on the remote terminal

RUN means that the command is coming from the remote keypad

Action: Press stop key

key

RDY is the state of the product

Action: Press local remote key

HMI is switching on the terminal