# How to Operate this Sample Program

## This sample includes

- SCADAPack Workbench project – Complete example showing how to use these two FBs: Word2Bit and Bit2Word. This is an export of a TCP project for any SCADAPack 300E. To import the entire project or to import just the FBs into your existing project, see the instructions below.

- E Configuration – This example for sp350E sets the IP address and enables the ISaGRAF/TCP server.

## Connections

- Connect your PC to your SCADAPack E RTU using an Ethernet connection.

- Connect USB cable from PC to RTU.

## Setup

- Run the E Configurator and open the configuration file included in this sample for a 350E, or create your own E configuration file:

- Edit the TCP/IP folder and set the RTU’s IP Address to an address compatible with your PC’s IP address. (e.g. PC = 192.168.2.100 and RTU = 192.168.2.1)

- Enable the ISaGRAF/TCP service

- Write the configuration over USB to RTU.

## Import Entire Project

To import just the new FBs into your existing project, see the next section “Import FBs Only”.

- Run SCADAPack Workbench

- From the Start Page, create a New Project of type SCADAPack 300E TCP/IP for an Ethernet connection to your PC.

- Give the project a name and select OK.

- Right-click the Project name shown in the Solution Explorer and select Import > Import Exchange File.

- Browse to the workbench file, Word2Bit_Bit2Word.7z, included in this sample and import. Select Cancel when done to exit the Import dialog.

- Open the Deployment page:

For Ethernet connections, select the ‘T’ connection graphic to open the LAN connection properties. Enter the IP Address of your RTU.

- Select Build > Clean Solution

- Select Build > Build Solution

- Download Solution to the RTU.

## Import FBs Only

- Run SCADAPack Workbench

- From the Start Page, open your existing project.

- Right-click the Resource name, not the Project, and select Import > Import Exchange File.

- Browse to the workbench file, Word2Bit_Bit2Word.7z, included in this sample and check only the FB(s) desired as shown below:

- Select import. Then select Cancel when done to exit the Import dialog.

- Select Build > Clean Solution

- Select Build > Build Solution

- Download Solution to the RTU.