# How to Operate this ClearSCADA Sample Program

### DISCLAIMER

These sample programs are intended to demonstrate one or more programming functions or methods and are presented as programming examples only. Schneider Electric assumes no liability for the use or application of these example programs or any portion thereof.

### Version Requirement:

You will need ClearSCADA 2013 R1 or newer to import this project.

### Setup:

Import the ClearSCADA project. You may import the project into an existing database or an empty database.

- Right click the database on the folder where you wish to import to, and select Import.

- After importing you must read and perform the instructions described on the four mimics shown below:

### Demonstration:

- This sample project provides a Lift station template and four instances of the template as shown circled below.

- Open the City Map mimic first as shown above. Zoom into the map to locate the lift stations and then select a lift station to view in detail.

- Instances 1 & 2 include an additional folder called Logic that provides simulated inputs and outputs so that these two lift stations can be demonstrated in simulated operation.

- Instances 3 & 4 are exact instances of the lift station template. To see these live, they require editing the outstation object and connecting to a live RTU.

- To put a lift station in service in a real application, you must edit the Outstation object shown in the example below for Instance 3.

- Other customization required for using this sample in a real application:

  - For each lift station instance, edit the 5000 IO object in the folder SCADA Equipment / RTU and select the IO Module type that matches your  SCADAPack type.

  - For each lift station instance, edit the Radio diagnostics object if using Trio radios. This object is found in the folder SCADA Equipment / Radio.

  - Replace the map mimic with a map of your application.

  - Add links to the map for each of your lift station instances.