UnityPro_V131_HF0465853R

ReadMe

# What is new with UnityPro_V131_HF0465853R

## Description of issue or behavior (before installation of UnityPro_V131_HF0465853R)

Some applications created using PME DTM Library V 1.0.23 were not working after migrating to new DTM version of V1.0.30. SCAIME (PME SWT 0100 module) .

Device function library expects DDT with UINT/UDINT datatype, but latest version of DTM was creating DDT with WORD/DWORD as datatype.

This caused the applications, built with v1.0.23, to result in build error

## New behavior after installation of UnityPro_V131_HF0465853R

DTM for Scaime (PME SWT 0100) will now provide datatype as UINT & UDINT instead of WORD & DWORD.

## What to do after installation of UnityPro_V131_HF0465853R

Update Unity Pro catalog.

## Compatibility

No compatibility break.

- Project done before Hot Fix HF0435420R installation can be open and built (stu, sta or zef) after UnityPro_V131_HF0465853R installation.

- Project done after UnityPro_V131_HF0465853R installation can be open and built (stu, sta or zef) with Unity Pro even if UnityPro_V131_HF0465853R is not installed

# Installation procedure

- Download attached file on your computer.

- Prior to installation make sure Unity Pro is closed.

- Run UnityPro_V131_HF0465853R to install the update

UnityPro_V131_HF0465853R can be installed over Unity V13.1

## Other updates to install prior to installation of  UnityPro_V131_HF0465853R

none

# How to know which hot fix or updates are already installed?

Open Unity Pro, click on “Help”, “about Unity Pro” and “Technical Information”:

The list of installed hot fixes is displayed.

# Important Notice: TüV certificate

This Hot Fix must not be installed on Unity Pro V13.1 XLS to generate Safety projects, as TüV certification status is not valid for this Hot Fix.

To check TüV certification status, go to : https://www.tuvasi.com/

Select « Certification Database »

Search for SCHNEIDER ELECTRIC certificate 01/205/5610/00/17

(Safety-related Programmable Electronic System   - M580 Functional Safety Controller System)

Select “View Details”

Open revision list in Appendixes.