ControlExpert_V150_HF0380584E_B

ReadMe

# Content

This Hot Fix improves the behaviour for the listed issues below:

| Internal Number | Description |
| --- | --- |
| PEP0644900R | Change behavior when project setting « Maintain output links on disabled EF (EN=0)” is not selected |
| TFS68457 | Add a CIP Safety DTM on a locked participant is possible |
| PEP0652062R | CE15-Adding NOC401 to M340 app crashes CE |

# PEP0644900R: Change behavior when project setting « maintain output links on disabled EF (EN=0)” is disabled

Control Expert V15.0, Program-Safe part using Safety EF on Ladder and FBD languages and Project Settings Option “Maintain output links on disabled EF (EN=0)” is not selected:

This option is adjustable in menu Tools/Project Settings/Program/Common.

## Description of issue PEP0644900R or behavior (before installation of ControlExpert_V150_HF0380584E_B)

For Safety EFs, management to the output links is different from CE V15.0 versus previous Control Expert versions. This is only true when option “Maintain output links on disabled EF (EN=0)” is not selected (option by default).

When the EN input of the safety EF is disabled (EN =0), its output link keeps its previous values even if it should be set to 0.

Example using Control Expert V15.0 when “Maintain output links on disabled EF (EN=0)” option is Not selected:

EN is set to 0 but output link OUT is still activated, EF linked to OUT stays active and S_ADD_INT operation is still done.

Behavior expected: Output link becomes red (inactive) and S_ADD_INT so.

NOTA: 
When option “Maintain output links on disabled EF (EN=0)” is selected behavior is conformed to what it is expected. (When the EN input of the safety EF is disabled (EN =0), its output link keeps its previous values)

Default described above is not true for Programs-Process tasks.

## New behavior after installation of ControlExpert_V150_HF0380584E_B

New behavior after HF installation:

Application part behave like it was previously using Control Expert V14.1 and previous versions.

Output links will be definitively maintained or set to 0 on a deactivated EF as per the chosen Project Settings Option “maintain output links on disabled EF (EN=0)”.

## What to do after installation of ControlExpert_V150_HF0380584E_B

## / Compatibility

Please make a Rebuild All Project

## Compatibility

No compatibility break.

- Project done before ControlExpert_V150_HF0380584E_B installation can be open and built (stu, sta or zef) after ControlExpert_V150_HF0380584E_B installation.

- Project done after ControlExpert_V150_HF0380584E_B installation can be open and built (stu, sta or zef) with Control Expert even if ControlExpert_V150_HF0380584E_B is not installed

# TFS68457: Add a CIP Safety DTM on a locked participant is possible

## Description of issue TFS68457or behavior (before installation of ControlExpert_V150_HF0380584E_B)

Conditions to observe this behavior:

-Create a CE V15.0 Safety application using a CPU BMEP58x040S

-Configure at least one safety discrete module on main rack and on ERIO rack, from the DTM browser

-Add a CIP Safety DTM.

-Enable Protection active option for Safety program from the Properties of Project configuration screen, select Program & Safety Protection tab

-Complete Password by clicking “Change password...” button

This done it should be no more possible to:

- add, copy/paste or delete a Safety module,

- add, copy/paste, rename, delete a CIP Safety DTM

- change the CIP Safety DTM’s IP address.

When trying any of the actions listed above, the message below is displayed :

Issue description:

From the DTM Browser it is still possible to add, to copy/paste, to rename, to delete a CIP Safety DTM and to change its IP address.

Trying to do this, the previous message always appears, even twice, but at the end the Safety DTM is added, copy/paste, renamed or deleted depending actions you try. It is possible to change the CIP Safety DTM’s IP address.

## New behavior after installation of ControlExpert_V150_HF0380584E_B

New behavior after HF installation, it is no more possible to add (CIP Safety DTM no more present on DTM list) or to copy/paste, delete (options grayed), to rename (Error Diag Box) a CIP Safety DTM and to change the CIP Safety DTM’s IP address (Error Diag Box) on protected application.

## What to do after installation of ControlExpert_V150_HF0380584E_B

## / Compatibility

No action required.

## Compatibility

No compatibility break.

- Project done before ControlExpert_V150_HF0380584E_B installation can be open and built (stu, sta or zef) after ControlExpert_V150_HF0380584E_B installation.

- Project done after ControlExpert_V150_HF0380584E_B installation can be open and built (stu, sta or zef) with Control Expert even if ControlExpert_V150_HF0380584E_B is not installed

# PEP0652062R: CE15-Adding NOC401 to M340 app crashes CE

## Description of issue PEP0644900R or behavior (before installation of ControlExpert_V150_HF0380584E_B)

Issue occurs when the HF ControlExpert_V150_HF0380584E is installed and is linked to the correction done to solve issue reported by the ticket TFS 68457.

Adding an Ethernet module in the rack (BMXNOC040x, 140NOC78xxx ) drives to an unexpected Control Expert closure.

One of the ways to reproduce the issue, using CE V15.0, Hot Fixes ControlExpert_V150_HF0380584E, ControlExpert_V150_HF_Integrity_Check:

- Create a new M340 application using a CPU BMXP3420302.

- Rebuild All the Project

- Configure a BMXNOC0401.2

Error message is displayed, module is not added:

- Open the DTM browser -> BMXNOC0401 is present.

- Close the project

- Error message is displayed

For all modules configured in the rack and for which part of the configuration is done via a DTM from the DTM browser for PLC range M580, M340 and Quantum.

M580 : BMXNOC0402

M340 : BMXNOC0401.2

Quantum : 140NOC78100, 140NOC78000

## New behavior after installation of ControlExpert_V150_HF0380584E_B

New behavior after HF installation, it is possible to add Ethernet Module Inrack (BMXNOC040x, 140NOC78xxx )

## What to do after installation of ControlExpert_V150_HF0380584E_B

## / Compatibility

No action required

## Compatibility

No compatibility break.

- Project done before ControlExpert_V150_HF0380584E_B installation can be open and built (stu, sta or zef) after ControlExpert_V150_HF0380584E_B installation.

- Project done after ControlExpert_V150_HF0380584E_B installation can be open and built (stu, sta or zef) with Control Expert even if ControlExpert_V150_HF0380584E_B is not installed

# Hot Fix Installation procedure

- Download attached file on your computer.

- Prior to installation make sure Control Expert is closed.

- Run ControlExpert_V150_HF0380584E_B to install the update

ControlExpert_V150_HF0380584E_B can be installed over Control Expert V15.0

# Other updates to install prior to installation of  ControlExpert_V150_HF0380584E_B

| Hot Fix Name | Link on SE.com |
| --- | --- |
| ControlExpert_V150_HF_Integrity_Check | https://www.se.com/fr/fr/download/document/ControlExpert_V150_HFIntegrity/ |

## How to know which hot fix or updates are already installed?

Open Control Expert, click on “Help”, “about Control Expert” and “Technical Information”:

The list of installed hot fixes is displayed.

# Important Notice: TüV certificate

Before installing this Hot Fix on EcoStruxure Control Expert to generate Functional Safety projects, you must check the TüV certification status on web site:

Go to : https://www.certipedia.com/

In Search tool prompt “M580”, then select SCHNEIDER ELECTRIC certificate 01/205/5610/01/19

Or look for SCHNEIDER ELECTRIC certificate 01/205/5610/01/19    (Safety-related Programmable Electronic System   - M580 Functional Safety Controller System) in certification database.

Select “View Details”

Open revision list in Appendixes and check that ControlExpert_V150_HF0380584E_B is part of the list.