# System requirements

System with Microsoft Windows 7 Professional 32/64bits

System: Pentium Processor 2.4 GHz or higher, recommended 3.0 GHz
RAM memory: 2GB minimum; recommended: 3GB
RAM memory (using DTM or X80 drops) : 2GB; recommended: 4GB
Hard disk: minimum available free space 8 GB, recommended 20 GB
Microsoft Internet Explorer 5.5 or higher

On Windows 7 64bits System, installation of Windows 7 Service Pack 1 (SP1) is mandatory in order to use Unity Pro V8.1

Windows 7 Professional 64bits , with a minimum 4GB RAM memory is recommended for projects using more than 20 DTMs.

System with Microsoft Windows 8.1

System: Pentium Processor 2.4 GHz or higher, recommended 3.0 GHz
RAM memory: 2GB minimum; recommended: 3GB
RAM memory (using DTM or X80 drops) : 2GB; recommended: 4GB
Hard disk: minimum available free space 8 GB, recommended 20 GB

Important Notice : Unity Pro can only run in “Windows 7 compatibility mode”

To set “Windows 7 compatibility mode” :

After installation of Unity Pro, start  UPWin8.exe. This exe sets Unity Pro exe files (UnityX.exe, CfCatGui.exe, LibsetServer.exe, SETUPEF.exe, LibsetServer.exe)  in the Windows 7 compatibility mode

- Note :   It works only for "current user".

Other method is :

- right click on Unity Pro icon (or UnityX.exe)

- edit Properties

- In Folder Compatibility, select “run  this program in windows 7 Compatibility mode

Same operation must be done on following exe files :

- CfCatGui.exe, LibsetServer.exe, SETUPEF.exe   (available in ..\Program Files\Schneider Electric\Unity Pro)

- LibsetServer.exe (available in ..\Program Files\Schneider Electric\Unity Pro\LibsetServer)

System with Microsoft Windows Server 2008

Recommended version : Windows Server 2008 R2, standard version, SP1

Important notice

Unity Pro requires .NET 3.5. Depending on the target Windows version and Windows configuration, .Net 3.5 may be

- not installed

- installed but not enabled ( the related "windows feature"  is not set ON )

- installed and enabled

Unity Pro setup  tries to automatically install and enable the required .Net 3.5 on the target Windows if necessary.

On Windows 8  and Windows Server 2008, automatic or manual installation of .Net 3.5 may require an internet connection.

If automatic  installation of  .Net 3.5  fails, please read & follow  MicroSoft technical notes on .NET 3.5 installation:

- Installing the .NET Framework 3.5 on Windows 8 or 8.1

http://msdn.microsoft.com/en-us/library/hh506443.aspx

- Microsoft .NET Framework 3.5 Deployment Considerations

http://go.microsoft.com/fwlink/p/?LinkId=256570